#### Roll No:		    Name:

#### 20MEP36 Design of Jigs, Fixture and Press Tools

Quiz-1

1) Process of developing tools, methods and techniques to improve productivity is known as

A) Machine design 	B) Engine design

C) Tool design	 	D) Pump design

2) The device in which a component is held, located for a specific operation and guides one or more cutting tools during machining is known as

A) Fixture                    B) Gauge

C) Template    	D) Jig

3) The device in which a component is held, located for a specific operation and does not guide cutting tools during machining is known as

A) Fixture 	B) Gauge

C) Template    D) Jig

4) The locator used to locate cylindrical components is

A) V blocks    B) Jig bushes

C) Pins           D) Surface plates

5) Location and guiding of cutting tools such as twist drills and reamers were done by

A) V blocks 	B) Jig bushes

C) Pins            D) Surface plates

6) Clamping should always be arranged

A) Directly above the points of supporting the work

B) Directly below the points of supporting the work

C) Directly above the points of supporting the tool

D) Directly on the cutting tool

7) The location which is to be used if the diameter of hole is in considerable variation is

A) Cylindrical location	 B)  V block location

C) Pin location		D) Conical location

8) The application of jigs and fixtures

A) Decreases	the production

B) Increases the production

C) Requires skilled labour

D) Decrease the accuracy in the components

9) The main required property in the selection of material for locating pins used in the jigs and fixture is

A) Shear strength B) Tensile strength

C) Wear resistance D) Brittleness

10) Devices designed for locating and holding cutting tools are called as

A) Fixtures     B) Tool holders

C) Jigs	           D) Gauges

11) In jigs and fixtures, holding and securing the work piece in the located position was done by

A) Clamping elements B) Bush

C) Locating elements	D) Button

12) In milling fixtures, position of the base on the table is accurately located by means of

A) Clamp               B) Bush

C) Tenon strips	D) Button

13) Length of the tenon strip is

A) Twice its depth B) Twice its width

C) Thrice its width  D) Thrice its depth

14) Tenon strips are made of

A) Cast iron, case hardened and ground

B) Stainless steel, case hardened and ground

C) Carbon steel, case hardened and ground

D) Aluminum, case hardened and ground

15) Two tenons should be kept --------------in the one of the TEE slots of the machine table

A) As close as each other

B) As apart as each other

C) Touching each other

D)   Overlapping each other

16) Jig is used for

A) Drilling, reaming and tapping

B) Milling  and grinding

C) Shaping and turning

D) Honing and lapping

17) Fixture is used for

A) Drilling, 	B) Reaming

C) Tapping  	D) Broaching

18) Clamp which holds the blank during machining need  to be

A) Loose 	B) Strong and rigid

C) Weak 	D) Broaching

19) The number of degrees of freedom for any work piece in space are

A) 12	B) 3   C) 6  D) 1

20) The minimum number of locators required to locate the workpiece are

A) 2   	B) 3  	C) 6  	D) 1

1.C

2.D

3.A

4.A

5.B

6.A

7.D

8.B

9.C

10.B

11.C

12.C

13.B

14.C

15.B

16.A

17.D

18.B

19.A

20.A