#### Roll No:		    Name:

#### 20MEP36 Design of Jigs, Fixture and Press Tools

Quiz-2

1. Locating pins of longer length are called

A) Shaft 	B) Set screws

C) Plugs 	D) Bush

2. Buttons are used for

A) Vertical location 	B) Horizontal location

C) Inclined location

D) Semi angular  location

3. Locating pins are generally used in fixtures for

A) Vertical location 	B) Horizontal location

C) Inclined location

D) Semi angular location

4. The best method to locate a rough surface is to use

A) Angular locators 	B) Horizontal locators

C) Relieved locators 	D) Edge locators

5. Drill bushes are made of

A) High speed steel 	B) Low carbon steel

C) High carbon steel, hardened and ground

D) Aluminium

6. Tenon strips are mounted to the base of milling fixture for the purpose of

A) Locating the work piece

B) Clamping the work piece

C) Guiding the cutting tool

D) Aligning the fixture on the machine table

7. For stable location of a flat surface , the number of rest buttons required is

A) 2  	B) 3  	C) 5  	D) 7

8.The 3-2-1 scheme of location using rest  pads is applicable to the location of

A) Solid cylindrical parts

B) Cylindrical parts with through hole

C) Cylindrical parts with center  hole

D)  Prismatic parts

9.  When a cylindrical work piece is placed on a V block , it is deprived of the following number of degrees of freedom

A) 2  	B) 3  	C) 5  	D) 4

10. A V block with 4 rest pins are used for location of

A) Short cylindrical parts

B) Finish machined long cylindrical parts

C) Unmachined cylindrical parts

D) Prismatic parts

11. In shaft-basis system, the basis shaft is one

    - whose upper deviation is zero

    - whose upper and lower deviations are zero

    - whose lower deviation is zero

    - none of the above

  - 12. According to Indian standard, 50 H8-g7 means

    - upper limit is (50+8) mm and lower limit (50-7) mm

    - designation of tolerance with basic size 50 mm

    - designation of fit of two parts with basic size 50 mm

    - none of above

  - 13. According to Indian standard, total number of tolerance grades are

(a) 10   (b) 2  (c) 18  (d) 8

  - 14. According to Indian standard, 50 H8-g7 means

    - tolerance grade for hole is 8 and for shaft is 7

    - tolerance grade for shaft is 8 and for hole is 7

    - designation of fit on shaft-basis system

    - none of above

15. Jigs and Fixtures are used for

(A) Mass production

(B) Identical parts production

(C) Both ‘A’ and ‘B’

(D) None of the above

16. The use of jigs and fixtures

(A) Facilitates the deployment of less-skilled labor for the production

(B) Eliminates pre-machining operations like marking, measuring, laying out, etc.

(C) reduced manual handling operations

(D) All of the above

17. The following is (are) the function(s) of a jig

(A) Holding

(B) Locating

(C) Guiding

(D) All of the above

18. A fixture does not

(A) Holds the workpiece

(B) Locate the workpiece

(C) Guide the tool

(D) All of the above

19. Jigs are not used in

(A) Drilling

(B) Reaming

(C) Tapping

(D) Milling

20. Fixtures are used in

(A) Milling

(B) Shaping

(C) Turning

(D) All of the above

1.C

2.B

3.A

4.C

5.C

6.D

7.B

8.D

9.B

10.A

11.C

12.B

13.C

14.A

15.A

16.A

17.D

18.C

19.D

20.D