#### Roll No:		 Name:

#### 20MEP36 Design of Jigs, Fixture and Press Tools

Quiz-3

1. Which one of the following is used to guide the tool and hold the job in mass production?

a) Gauge b) Housing

c) Jig       d)Fixture

2.  Which one of the following is used to clamp the job in relation to the tool in mass production?

a) Gauge b) Edge clamping

c) Jig d)Fixture

3) Which one of the following device  is used to fabricate a job by welding which is set in this device so that it can be swiveled around 3600  as per requirement?

a) Turning Fixture   b) Welding Fixture

c) Broaching Fixture d) Boring Fixture

4) Usually drill jig is not clamped to the drilling machine table. Which among the following for this reason for this?

a) It is rigid the this operation

b) It is easy for the operation

c) Number of holes of various sizes are being drilled in different settings

d) It is a more time consuming device

5) Which among the following locators is the best location of a round shaped  component?

a) Pin type locator

b) Wedge type locator

c) Vee Locator

d) Adjustable stop locator

6) Which among the following is the purpose of providing bushings in a drill jig?

a) For easy drilling

b) For determining the size of the hole to be drilled

c) For locating accurately and guiding the drill for precise drilling operation

d) For getting good finished surface in the drilled holes

7)Drill jig bushings are generally made of

a) Mild Steel b) Cast steel

c) Cast Iron d) Tool steel

8) Drill bushings are normally hardened to

a) Protect the jig from damage

b) Ensure prolonged life without wear and tear so as to guide the tool accurately

c) Guide the tool so that it does not go inclined

d) Allow the chips to come out easily

9) Plain drill jig bushings are generally secured in the body of the jig so that the bushings should

a) Not vibrate, rotate and be withdrawn while in operation

b) Rotate when the tool is rotating

c) Vibrate while in operation

d) get withdrawn with the tool

10) The ________ bushings are used where multiple operations, such as drilling and reaming or drilling and tapping, are to be performed on the same hole

a) Slip renewable  b)  Fixed renewable

c) Oil-groove        d) Template

11. A jig is used to

    - drill a hole

    - locate and clamp a work piece

    - locate, clamp the work piece and guide the cutting tool

    - set the work piece for measurement

12. Common feature between jig and fixture is that both are used for

- cutting the work piece

- holding the work piece

- locating the work piece

- testing and inspection of the work piece

13. Slip bush is used in jig where

- the drill bit can slip on the work piece

- holes are too close

- the bush is to be replaced every time

- a liner is used along with bush

14. Liner in a bush of a jig

- saves the bush from wear

- strengthens the bush

- offers lubrication

- aligns the bush in a hole

15. A cam is used in a jig to

- locate the work piece

- lift a heavy work piece

- move the work piece frequently

- to clamp the work piece

16. A turning fixture is used on the lathe

- to turn a non‐symmetric job

- so that the surface finish of the work piece is not spoiled

- to turn the work piece which otherwise cannot be fitted on the lathe

- to turn long jobs

17. An indexing jig is used to

- plane surfaces

- one angular settings

- multi angular settings

- multi plane settings

#### 18. The jigs and fixtures can be constructed through

a. Casting      b. Fabrication

c. Welding    d. All of the above

19. The quickest clamping device is a

- wing nut  b. knurled nut

c. cam/eccentric  d. Conventional nut

20. A conical locater has the advantage of

- easy location

- self centering

- easy location and self‐centering

- offers good grid

1.C

2.B

3.B

4.C

5.C

6.A

7.A

8.B

9.A

10.A

11.A

12.C

13.C

14.A

15.D

16.A

17.C

18.D

19.C

20.B