#### Roll No:		    Name:

#### 20MEP36 Design of Jigs, Fixture and Press Tools

#### Quiz-4

#### 1.The operation of cutting of a flat sheet to the desired shape is called

(A) Shearing     (B) Piercing

(C) Punching    (D) Blanking

#### 2. Piercing is an operation of cutting

(A) a cylindrical hole in a sheet of metal by the punch and the die

(B) a hole (other than cylindrical) in a sheet of metal by the punch and the die

(C) a flat sheet to the desired shape

(D) a number of holes evenly spaced in a regular pattern on a sheet of metal

#### 3.In blanking operation, the clearance is provided on

(A) punch                (B) die

(C) half on the punch and half on the die

(D) either on a punch or die depending upon the designer's choice

#### 4.In piercing operation, the clearance is provided on

(A) punch                  (B) die

(C) half on the punch and half on the die

(D) either on a punch or die depending upon the designer's choice

#### 5.Punching a number of holes in a sheet is known as?

(A) Perforating    (B) Parting

(C) Notching        (D) Lancing

#### 6.Shearing the sheet into two or more pieces is known as?

(A) Perforating     (B) Parting

(C) Notching         (D) Lancing

#### 7.Removing the pieces from the edge in shearing operation is known as?

(A) Perforating      (B) Parting

(C) Notching         (D) Lancing

#### 8.Leaving a tab without removing any material is known as?

(A) Perforating      (B) Parting

(C) Notching         (D) Lancing

#### 9.Moving a small straight punch up and down rapidly into a die is done by a process known as?

(A) Perforating       (B) Parting

(C) Nibbling           (D) Lancing

#### 10.As the thickness of the sheet is increased the clearance needed will also?

(A) Increase           (B) Decrease

(C) No effect

(D) First decreases and then increase

#### 11.Bevelling is particularly suitable for shearing of?

(A) Thin blanks

(B) Thick blanks

(C) Very thin blanks

(D) Medium thin blanks

#### 12.As the clearance increases, the punch force required?

(A) Decreases

(B) Increases

(C) Remains the same

(D) First increases and then decreases

(D) As the clearance increases, the punch force required decreases. As the thickness of sheet is increased the clearance needed will also increase. Clearance needed is directly proportional to thickness of sheet.

#### 13.Lancing is the operation of

(A) cutting a sheet of metal in a straight line along the length

(B) removal of metal to the desired shape from the edge of a plate

(C) cutting a sheet of metal through part of its length and then bend the cut portion

(D) bending a sheet of metal along a curved axis

#### 14.Notching is the operation of

(A) cutting a sheet of metal in a straight line along the length

(B) removal of metal to the desired shape from the edge of a plate

(C) cutting a sheet of metal through part of its length and then bending the cut portion

(D) bending a sheet of metal along a curved axis

#### 15.The operation of straightening a curved sheet metal is known as

(A) drawing     (B) squeezing

(C) coining       (D) planishing

#### 16.The operation of bending a sheet of metal along a curved axis is known as

(A) plunging        (B) notching

(C) slitting           (D) forming

#### 17.during drawing operation, the states of stress in cup would include

(A) compressive stress in the flange

(B) tensile stress in the wall

(C) both (a) and (b)

(D) none of these

#### 18.The operation of producing cup-shaped parts from flat sheet metal blanks by bending and plastic flow of metal is known as

(A) drawing         (B) squeezing

(C) coining          (D) planishing

#### 19.Cutting and forming operations can be performed in a single operation in a

(A) simple die           (B) compound die

(C) combination die  (D) progressive die

#### 20.Which of the following methods of manufacturing is used for the production of appliances like the fridge and the vacuum cleaner?

(A) Forging (B) Deep drawing

(C) Sheet metal forming and cutting

(D) Rolling

1.B

2.A

3.A

4.B

5.A

6.B

7.B

8.D

9.C

10.A

11.B

12.B

13.C

14.B

15.D

16.C

17.A

18.A

19.B

20.C